COMMUNICATIONS ALLIANCE

Service Provider Guide for the ‘So you want a VoIP phone service?’ Communications Alliance Customer Booklet

Introduction

This Guide is a companion document to the Communications Alliance ‘So you want a VoIP phone service?’ Customer Booklet.

The Customer Booklet was developed to assist prospective (domestic and SOHO) users of VoIP services by providing a step-by-step process to help them decide upon a VoIP service, with a particular emphasis on the services and consumer protections provided to Australians who have current telephony services.

Who are these guidelines intended for?

This Guide provides assistance for VoIP Service Providers to help in considering the technical, regulatory and consumer protection issues that have been covered in the Customer Booklet and that prospective users may enquire about.

Individuals or organisations who are considering becoming users of VoIP services are recommended to read the ‘So you want a VoIP phone service?’ Customer Booklet, available at http://www.commsalliance.com.au/Activities/voip. This Guide provides further information for those who wish to learn more about VoIP services.

Feedback

This is a living document, which will be updated on a regular basis to reflect ongoing developments in the technical and regulatory environment. It is recognised that there are areas in this Guide where further information is required. Readers are encouraged to forward new information, suggestions and corrections to Mike Johns at m.johns@commsalliance.com.au to be incorporated into future releases.  Communications Alliance would like to encourage an open and interactive dialogue with members of the industry to assist the ongoing development of this Guide. Please note that information that is provider-specific cannot be included.

Contents

1	Service features and applications

Telephone line provisioning

Service features

Business needs

Other Needs

Access for people with disabilities

2	Telephone Numbers

Phone number provisioning

Making calls

Emergency access

Phone number displays

Number directories and assistance

3	Broadband service

Broadband suitability

Choosing a broadband service

4	VoIP service

Choosing a VoIP service provider

5	Customer premises

Configuration in customer’s premises

Quality

Availability / Power supply

General safety and privacy/security issues

6	Faults

How to go about solving faults and problems

7	Customer service

Customer service

8	Billing

Service charges

Contracts and billing

9	Consumer protection issues

Consumer protection

## How to use the information in this Guide

Issues concerning the provision of VoIP services have been divided into nine high-level topics. Each topic in turn provides guidance on specific items as described in the diagram below.

A comprehensive index is provided towards the back of this Guide.

References to further resources for the VoIP Service Provider

Background information for the VoIP Service Provider

Guidance to be offered to a prospective VoIP user depending on the context of the question

Examples of questions to ask a prospective VoIP user to determine what they are looking for and their level of knowledge

Thus is the page reference to the corresponding topic in the Customer Booklet

# Service features and applications

| ## Telephone line provisioning |  |
| --- | --- |
| Number of lines and calls Page 7 | Questions to ask How many phones lines do you use now? Do you want to be able to make phones calls while using the internet and emailing? How will you be using your VoIP service in the future? What phones will you want to use (your existing phones or purchase new phones)? Guidance to provide You should describe what services are provided with your account including distinctions between residential accounts and business accounts where multiple lines may be offered. You should describe the VoIP services being offered in terms of traditional ‘phone lines’ and how a VoIP service may differ. For example: - What telephone (‘indial’) number(s) come with your account? - How many concurrent services are being offered in the plan? |
| Keeping POTS Page 8 | Guidance to provide If the inquirer has a Digital Subscriber Line (DSL) service, you should explain that a DSL service requires the existing phone line to operate over. You can bring to their attention that their existing telephone provider may have more economical plans that could be considered in conjunction with using a VoIP service. If the inquirer has a cable or wireless broadband service, you can explain that these broadband services are independent of the existing phone line. You should bring to their attention that there are other benefits in retaining the existing phone line such as being able to use some phones (e.g. those that do not use mains power to work) in the case of a blackout. |

| ## Service features |  |
| --- | --- |
| Services Page 8 | Questions to ask What features do you currently use and what phone and other devices do you use? What additional features are you interested in? Guidance to provide You can inform the inquirer of other services that you are able to provide in conjunction with the VoIP service. They may already be familiar with some of these services in the context of similar services over the Public Switched Telephone Network (PSTN). You will need to explain that the correct operation of some services will depend on the type of Customer Equipment being used. You should inform the inquirer of the equipment that you support in conjunction with your services. You should provide information on these features on your website. Examples - voicemail messages (typical options to receive voicemail messages include accessing via a telephone, by logging in to the VoIP service provider’s website or as an attachment in an email) - call waiting, call forwarding, call barring, Calling Number Display, Calling Number Display blocking - conference calls - instant messaging (IM) - video calls - ability to send text or visual information during a call - higher quality voice calls - using your VoIP service wherever you are (nomadicity) - presence (status of your availability to take calls and the availability of the person you are calling) - sending and receiving faxes - ability to send and receive real-time text and video during a call |
| Equipment Page 8 | Guidance to provide You should list all the options that you provide in your plans, including: - VoIP programs on computers (not really the subject of the Customer Booklet) - ATAs and how their existing phone can be used - VoIP phones that can be connected directly to the broadband service The ability to use a service will also depend on the equipment used by the person that they are calling. |
| Dial tones Page 23 | Guidance to provide You should describe (and demonstrate if possible) the tones that are used with the service. Typical tones include dial, engaged and call waiting. |

| ## Business needs |  |
| --- | --- |
| Business services Page 12-13 | Questions to ask What is the type and size of your business (home office, small business, mobile office)? What telecommunications services are you currently using for your business? Guidance to provide You need to determine their business needs and to explain that their needs may require additional consideration, for example, a higher level of service reliability and a higher voice quality for their calls. Other needs could include: - multiple extension numbers - audio and video conferencing - call centre functionality - remote teleworking - presence in other locales - inter-office communications (for example free ‘on-net’ calls between offices) - facsimile - EPTPOS You can inform them of special business plans that you may offer. Resources VoIP for Small Businesses http://www.setel.com.au/site.php?id=383 Demystifying Business VoIP Services (for SMEs) http://www.marketclarity.com.au/freebies |
| Third party consultancies Page 12 | Questions to ask What are your current IT requirements for your business? Guidance to provide You need to determine whether there are particular needs that may not necessarily be VoIP-related but are communications-related that require external consultancy, e.g. - office networking (e.g. servers, data storage, cabling) - security, including firewalls and other intrusion prevention - IT support - Teleworking |
| Privacy and confidentiality Page 12 | Guidance to provide You need to explain that there are a number of privacy and security issues that may require consideration, including: - the display of phone numbers - security and privacy of voice on an IP network presence and location information |

| ## Other Needs |  |
| --- | --- |
| Fax machines Page 13 | Questions to ask What facsimile machine do you currently use? Guidance to provide You need to explain whether your service supports facsimile communications. You need to determine the type of facsimile machine being used and whether you support it. Background Facsimile machines require support of the G.711 codec or T.38 (Fax over IP) |
| Home banking Page 13 | Questions to ask Do you do home banking by telephone (or use other services where you need a touch tone phone)? Background info There are two issues: DTMF compatibility and security. Touch tone dialling using DTMF (dual tone, multi-frequency), for example to navigate menu systems, may not function correctly, depending on whether the service provider’s network supports touch-tone dialing. Banks provide security technologies with their internet banking such as SSL connections, encryption and digital certificates to protect customers’ information. Resources - Security (general internet banking security advice) ANZ www.anz.com.au/protectyourbanking/ Commonwealth www.commbank.com.au/NetBank/security.asp National www.nab.com.au/Personal_Finance/0,,83947,00.html Westpac www.westpac.com.au/internet/publish.nsf/Content/WI+Security |
| Home alarm systems Page 13 | Questions to ask What home alarm system do you have installed and is it remotely monitored by a security company? Guidance to provide You need to explain whether your service supports alarm diallers. You need to determine the type of alarm dialer being used: - systems where security companies regularly poll the installed alarm diallers, or - diallers that just operate when activated Background Alarm diallers require support for the G.711 codec. Resources Australian Security Industry Association (ASIAL) home security alarm information at www.asial.com.au/driver.asp?page=asial/consumer+information/security+overview/home+security+alarms |
| Digital set top boxes Page 13 | Questions to ask What cable television service do you subscribe to and is it connected to your telephone line? Guidance to provide VoIP services at present will generally not support the connection that the set top box requires to call back to the subscription television provider. To maintain the connection to the subscription television provider, a PSTN service is recommended. Background Digital set top boxes currently incorporate dial-up modems and the QoS, reliability and compression characteristics of VoIP services at present generally will not support a dial-up connection on the VoIP service. Technically it may be possible (use of the G.711 codec is required) but using a VoIP service for this connection is not recommended at present. There are no more analogue set top boxes connected to the cable networks as the analogue network has now been switched off as of the end of January 2007. The three major pay TV providers are Foxtel, Optus and Austar. In addition there are Neighbourhood Cable (Ballarat, Geelong and Mildura) and TransACT (ACT and Queanbeyan). |
| Medical monitoring services Page 13 | Questions to ask: Can you describe the medical monitoring service that do you use? Guidance to provide You need to explain whether your service supports medical monitoring services. Background Medical monitoring services require support of the G.711 codec. |
| Priority assistance Page 13 | Guidance to provide You need to explain that priority assistance is only provided in conjunction with a Standard Telephone Service. The inquirer should speak to their service provider who is providing priority assistance. Background Currently Telstra, AAPT and Primus offer priority assistance services. Telstra is the only carrier required to provide priority assistance services to its customers as a condition of its licence. Resources ACIF C609:2007 Priority Assistance for Life Threatening Medical Conditions Industry Code ACMA Priority Assistance information at http://internet.aca.gov.au/WEB/STANDARD//pc=PC_2038 |

| ## Access for people with disabilities |  |
| --- | --- |
| Textphone or teletypewriters (TTYs) Page 12 | Guidance to provide You need to explain that the ability for TTYs to be used in conjunction with a VoIP service will depend on the compatibility with the Analogue Telephone Adapter (ATA) being used. Background The current understanding is that TTY support is not generally provided on any currently available mass market ATAs. Alternate services such as Text over IP (ToIP) may fulfil a need in the future. |
| National Relay Service (NRS) Page 12 | Questions to ask What NRS services do you currently use? Background Voice calls to the NRS (‘speak and listen’ or ‘speech to speech relay’) are supported in the same manner as other VoIP calls. The quality of the call will be important. All other calls that use text communications: - ‘Type and read’ (text to voice’) - ‘Type and listen’ (‘hearing carry over’) - ‘Speak and read’ (‘voice carry over’) will have the same dependency on the ATA & TTY compatibility. From the middle of 2007 the NRS will be supplying an internet relay service. This service is similar to a relay service using a text telephone, but the user accesses the service via MSN and AOL messaging on a PC, laptop or 3G phone. Resources Refer to the information provided by The National Relay Service at www.relayservice.com.au/calloptions/ |

# Telephone Numbers

| ## Phone number provisioning |  |
| --- | --- |
| Assigning numbers Page 11 | Questions to ask Are you looking to replace your existing phone service or are you planning to use a VoIP service alongside of your existing service? Guidance to provide You should explain what number comes with the VoIP service, how it can be used and how it differs from a PSTN (geographic) number. Background Options may include keeping your existing number or getting a new VoIP number. New numbers may be like your existing number or a ‘virtual’ (or ‘private’) number. Ask the VoIP service provider what is offered and what the differences are. A new ‘Location independent communications service’ number range (0550) has been introduced by ACMA as a part of the Numbering Plan for predominantly nomadic services. Resources ACMA Numbering information at www.acma.gov.au/WEB/STANDARD//pc=IND_TECH_TEL_NUM_OVIEW ACIF G610:2007 IPND Data Industry Guideline |
| Porting Page 11 | Questions to ask What phone service(s) do you have now and what are you planning/inquiring to change? Guidance to provide You should explain the processes that are in place for transferring their number (‘porting’) from one service provider to another and the conditions under which it can happen (including charges and timing). Currently the interest is in porting a PSTN number for use on a VoIP service. In the future the issue of porting between VoIP service providers and also from a VoIP service Provider back to a provider of a PSTN service will become relevant. The degree in difficulty in porting will also depend on whether they are simple or complex ports. Background Porting occurs between carriers networks who have a Ported Local Number Register (PLNR) so that other carriers can direct call traffic for the Ported Telephone Numbers to the new network. Carriers need to have porting and interface agreements and connections in place for ports to their network as well as ports away from their network. Carriers also need to be able to download every other carrier’s PLNR in order to determine the correct network for call routing to Ported Telephone Numbers. Resources ACIF C540:2006 Local Number Portability Industry Code ACIF G602:2006 Local Number Portability IT Specifications and Operations Manual ACIF G603:2004 Local Number Portability IT Test Strategy ACIF G613:2004 Local Number Portability IT Test Plan ACMA Local Number Portability information at www.acma.gov.au/WEB/STANDARD//pc=PC_1694 |

| ## Making calls |  |
| --- | --- |
| What numbers can be dialled? Page 11-12 | Guidance to provide You need to provide a ‘laundry list’ of numbers that are available and any difference from their current phone service. Numbers include local, long distance, mobile, international numbers, ‘13’ and ‘1300’ local rate numbers, ‘1800’ free phone numbers, ‘1900’ premium rate numbers. Background It is currently understood that there are no VoIP Service providers offering 1900 services due to the fact that call charging for these services cannot be done in real time. ‘13’ services (such as pizza deliveries and taxis) will not generally recognise your location. Resources ACMA Number Plan information at www.dcita.gov.au/communications_and_technology/policy_and_legislation/numbering/numbering_plan |
| On-net calls Page 12 | Questions to ask You need to explain the differences between ‘on-net’ calls and calls terminating outside of your network on traditional networks and other VoIP networks. Background Generally, VoIP Service Providers have not provided the ability for their customers to call numbers of other VoIP providers. |
| Nomadicity Page 12 | Guidance to provide You can explain that using the service from other locations (nomadicity) is dependant on the following factors: - what equipment is needed to use a service from another location - what conditions need to be present to be able to access your service You need to explain that the Emergency Service Operator will not necessarily know the location of a caller making an emergency call using a nomadic service. Background It may be possible for a customer to use their phone (with supporting equipment) and the service wherever they can get access to the internet. |
| Pre-selection Page 12 | Guidance to provide You need to explain whether you can supply pre-selection or not. Alternatively, for a user to be able to choose among multiple VoIP Service Providers, this would depend on whether their customer equipment could support multiple connections Background Pre-selection was originally introduced to provide consumers with a choice of providers without having to dial over-ride codes. It requires having existing relationships between providers to handover calls. For a customer using a VoIP service, the choice would be to have multiple VoIP providers and/or the ability to revert back to their PSTN line. This can be achieved through the use of ‘dial plans’ if the modem supports multiple VoIP providers. Examples where this could be used is where alternate providers offer better international call rates or the customer may wish to revert back to the PSTN for a 000 emergency call. Resources ACIF C515:2005 Pre-selection Industry Code ACMA Pre-selection information at www.acma.gov.au/web/STANDARD//pc%3DPC_2546 |

| ## Emergency access |  |
| --- | --- |
| Dialling ‘000’ Page 10 | Guidance to provide You need to explain whether you provide direct access to the emergency service operators. If you do, then you need to explain that the instructions of the ‘000’ and ‘106’ operators need to be followed. Background Part of providing a standard telephone service is the ability to meet the obligations under the Emergency Call Service Regulations. Resources ACMA information on Emergency Call Services www.acma.gov.au/web/STANDARD//pc%3DPC_2025 ACMA information on Internet calls (VoIP) http://internet.aca.gov.au/WEB/STANDARD//pc=PC_100550 ACMA information on VoIP Regulation http://www.acma.gov.au/WEB/STANDARD//pc=PC_310067 ACIF C536:2003 Emergency Call Services Requirements Industry Code |
| Location identification Page 10 | Guidance to provide You need to explain that it is unlikely that emergency call operator or the emergency service organisations (Police, Fire and Ambulance) will be able to identify where you are just from your number and that you need to verbally provide your location to them when asked. Background ACIF G629:2006 Interim VoIP Location Indicator for Emergency Services Signalling Specification |
| Real time ‘106’ text calls Page 10 | Guidance to provide ‘106’ is the emergency number provided by the National Relay Service (NRS) for those who are Deaf or have a hearing, speech or communication impairment. TTYs can only be used with compatible ATAs in order for a text call to be successful. |

| ## Phone number displays |  |
| --- | --- |
| Calling Number Display Page 12 | Questions to ask Do you currently have calling number display? Do you wish to prevent others from seeing your telephone number? Is your existing number an unlisted number? Guidance to provide The call information that accompanies an unlisted number does not automatically get transferred with the call when being handed from one network to another. You need to explain: - how incoming numbers will be displayed. - how to prevent others from seeing their number. - whether their number is to be a listed or an unlisted number. An unlisted number is also known as a ‘silent line’ (as provided by Telstra) Associated Issue When porting a number, the associated services, such as an unlisted number or silent line, are not necessarily ported along with the number. As a service provider, if you are the recipient of an unlisted number from another network, you are required to maintain the call information to preserve the number’s anonymity. The ACIF C522 Industry Code provides the CSP obligations for Calling Number Display. Consideration should be given for both on-net and off-net calls. Resources ACIF C522:2007 Calling Number Display (CND) Industry Code |

| ## Number directories and assistance |  |
| --- | --- |
| Directory assistance and operator-assisted calls Page 23 | Guidance to provide You need to explain if you provide directory assistance (either via a website or a human operator) and operator-assisted calls? Resources ACMA Directory assistance & associated services fact sheet http://www.acma.gov.au/WEB/STANDARD//pc=PC_2947 |
| Directory listings Page 23 | Questions to ask If the inquirer is planning to use their existing number for the VoIP service, you need to ask: Is your existing service listed, unlisted or is it a suppressed address? Background When porting a number, the associated services, such as directory listings, are not necessarily ported along with the number. Resources ACIF C555:2007 Integrated Public Number Database (IPND) Industry Code |

# Broadband service

| ## Broadband suitability |  |
| --- | --- |
| Speed Page 14 | Questions to ask Do you have broadband? If so, what type of broadband (ADSL, cable, wireless or satellite) and what plan are you on (speed and monthly data usage allowance)? What is your computer and broadband usage like at present? Advice – for DSL broadband You should explain the relationship of the VoIP service that you are providing and the bandwidth that it consumes in their ISP’s broadband plan. You should warn that if this limit is exceeded, calls will start to sound like ‘mobile’ calls (or may not be able to be made at all depending on the broadband plan). You can explain that there are several factors that can impact on the quality of their VoIP service by the choice of broadband service, including: - the upload and download speed of the connection. The speed of the service may be impacted by various factors including concurrent internet browsing, file uploading/downloading, the number of active users on the broadband service and possible electrical interference. You could also explain that there are factors beyond their control, including: - how far they live from the telephone exchange - the quality of the line connecting the exchange to their premises You can explain what VoIP services can typically be provided over different broadband plans. Typically for ADSL plans: - 256 / 64 kbps - is barely adequate for one call at a time (with no other computer activity) - 512 / 256 kbps - is very usable for residential usage - 512 / 512 kbps or more - would suit a small office with maybe five to ten phones Background The actual speed achieved for DSL services will depend on many factors, including the distance from the telephone exchange to the user’s premises, the line quality, electrical interference and the configuration of the user’s network and equipment. Seldom are the theoretical maximum speeds of the broadband service achievable. ISPs must comply with the Trade Practices Act when advertising broadband speeds. The actual speed achieved for cable will mostly depend on how many other users are active at the same time. The effects of latency of a satellite access link can cause a greater delay of VoIP packets compared to other access technologies. There are several hosted speed tests to determine the upload and download rates, some of which the server can be nominated, including: |
| Speed test Page 14, 17 | Guidance to provide A useful website called ‘TestYourVoIP’ at www.testyourvoip.com will make a VoIP call from wherever you are to one of several test locations and report the results. This test can be carried out also while downloading or uploading a file to the internet to illustrate the impact on your VoIP call. Resources Whirlpool www.whirlpool.net.au (a technical website on Australian broadband, including information on VoIP) Broadband speed tests: Oz Broadband Speed Test www.ozspeedtest.com Internet Connection Center www.pcpitstop.com/internet Ookla Speed Test www.speedtest.net |
| Data usage Page 15-16 | Guidance to provide You need to explain how making VoIP calls will use part of their monthly broadband data quota. You can explain that it is typical for ISPs to only count downloads in a customer’s broadband allowance (which represents ONE side of the conversation). If an ISP counts both uploads and downloads (representing BOTH sides of the conversation) then that customer’s data quota is used up at twice the rate. You can let them know if you allow your customers to control codec selection so they can choose the speech quality of their own calls (generally in order to reduce the impact on their download data quota). You need to let them know if you cap or throttle the broadband speed in the plan if a customer exceeds their monthly data usage. The result is either that customer will not be able to make further VoIP calls or alternatively the VoIP calls could increase their broadband bill. If you offer an ‘unlimited’ plan, you need to explain what features of the plan are unlimited. Background VoIP calls can typically use 10 MB to 40 MB (approximate figures) from the download quota for every hour of calls. Resources VoIP plan comparison at vOip chOice: www.voipchoice.com.au or at Oz Net Phones at www.ozinternetphones.com |
| Broadband and VoIP services Page 16 | Guidance to provide If you are the ISP supplying the VoIP service as a bundled product, you need to consider the following issues: - the Quality of Service (QoS) that you are providing - the phone numbers that your customers can and cannot ring? If you are solely a VoIP service provider, you need to consider the following issues: - your relationship with the broadband service provider - is your VoIP service treated equally on the ISPs network? |
| Assurance of service Page 17 | Guidance to provide If you are also the ISP, you need to explain what assurance of service you can provide as the broadband provider. Issues to consider are: - the availability of the service - the Quality of the service |

| ## Choosing a broadband service |  |
| --- | --- |
| Suitability Page 16 | Questions to ask Have you gone through the suggestions in the VoIP Customer Booklet including going to www.choice.com.au and www.broadbandchoice.com.au? Guidance to provide You may be able to assist them in selecting the type of broadband service that is best for their needs (ADSL, cable, wireless, satellite). You should refer users to an Internet Service Provider (ISP) who can provide suitable speeds and Quality of Service (QoS). Resources DCITA Broadbandnow website www.broadbandnow.gov.au Broadband comparison at Internet Choice at www.internetchoice.com.au |
| Availability Page 16 | Guidance to provide You can recommend ISPs to them that you have business relations with or alternatively send them to broadband CHOICE at www.broadbandchoice.com.au |

# VoIP service

| ## Choosing a VoIP service provider |  |
| --- | --- |
| Trial periods Page 18 | Guidance to provide You need to explain consumer rights, including statutory cooling-off periods in contracts Resources Refer to the ACIF C620:2005 Consumer Contracts Industry Code |
| Assurance of service Page 18 | Guidance to provide As the VoIP Service provider, you need to explain what assurance of service you can provide. Issues to consider are: - the availability of the service - the Quality of the service |
| Domicile of VoIP Service Provider Page 18-19 | Guidance to provide If you are based in Australia, you can provide them with an ABN number to demonstrate that you are an Australian company. You need to explain how the characteristics of the voice call may be affected under certain conditions, such as delay, echo and drop-outs. If you are based overseas, you need to explain how calls to emergency services (‘000’ and ‘106’ calls) are handled. If you are based overseas, then you need to explain how you bill customers, including the handling of inquiries and dispute resolution, and any specific aspects of the bills such as whether the bills will be in Australia dollars and if not, would they be subject to currency fluctuations. Background The Quality of Service (or QoS) is dependant on several factors. If the service is being provided from an overseas location, there would be some extra delay in the conversation and the quality may suffer. Customer contracts are dependant on which country the service provider’s business is located. Resources Communications Alliance An introduction to Consumer Codes www.commsalliance.com.au/about_us/factsheets ACIF C521:2004 Customer Information on Prices, Terms and Conditions Industry Code ACIF C542:2003 Billing Industry Code ACIF C547:2004 Complaint Handling Industry Code ACIF C620:2005 Consumer Contracts Industry Code Codes can be found at www.commsalliance.com.au/documents/codes ACMA Code information www.acma.gov.au/WEB/STANDARD//pc=PC_2524 |

# Customer premises

| ## Configuration in customer’s premises |  |
| --- | --- |
| Page 20 | Questions to ask What equipment do you already have which you intend to use for VoIP? What do you already know about connecting up a VoIP service? What do you need (or want) to know about connecting up a VoIP service? Where will you want to use the phones in your home? Guidance to provide You need to establish the inquirer’s competence in working with computers and telephony. You need to provide the following information as applicable: - the suitability of the customer's equipment for the VoIP service, especially whether their existing broadband modem has the necessary capabilities for handling VoIP and whether there are any restrictions on equipment or software. - the equipment that you supply for the VoIP service. - details of support you provide for (i) equipment you have provided; and (ii) equipment supplied by the customer. - where to seek assistance/information on setting up equipment for VoIP (other than your support services). - how to obtain responses to frequently asked questions (e.g. your website, email, customer hotline) Issues that may need to be considered include: - whether the VoIP service will work alongside their existing phone service - whether they need to purchase software or extra equipment such as an ATA for their existing phone - whether they wish to continue to use their existing handset - the location of the equipment and the telephone outlet - whether they wish to have a PSTN service to dial the emergency service operator in case the broadband or VoIP service becomes unavailable. Background Equipment currently owned by customer may have been locked by a previous provider for use on the previous provider’ network. Also there may be limitations with older equipment that prevent certain configurations or the support of some services. |

| ## Quality |  |
| --- | --- |
| Characteristics Page 9, 20 | Guidance to provide You need to explain the factors that can affect voice quality, including: - how their home network prioritises their calls. - how the VoIP Service Provider’s network may prioritise the calls. - how the ISP may prioritise the calls - How many calls does the user have in progress at the same time in relation to the capabilities of the broadband service? - other activities such as the uploading or downloading of files at the same time of the call? - once the call is beyond your network, the call quality is subject to other service providers’ networks and the call-terminating parties’ setup. You can explain that there may also be incompatibilities between different pieces of customers’ equipment (and software). Background Prioritisation of VoIP calls, both intentionally or unintentionally (e.g. via packet inspection) is an issue that has been raised by industry. Resources Communications Alliance has two active projects on QoS. See www.commsalliance.com.au/Activities/voip |
| Setup and optimising Page 20 | Questions to ask What equipment do you already have which you intend to use for VoIP? What do you already know about connecting up a VoIP service? What do you need (or want) to know about connecting up a VoIP service? Where will you want to use the phones in your home? Guidance to provide You need to clarify what equipment you support and the level of support that you provide. For the equipment that you do support you need to provide them with sufficient details to optimally set up their service. The customer needs to be advised of the trade-offs of keeping older equipment against investing in newer equipment. The customer needs also needs to be advised of that the quality of the handset can have an effect on the quality of a call. Background Consumer modem/routers are entering the market as ‘VoIP-ready’ devices incorporating support for packet prioritisation. Multifunctional devices may also include ATAs with ports for PSTN phones on the devices. |

| ## Availability / Power supply |  |
| --- | --- |
| Blackouts and loss of power Page 9 | Questions to ask Do you experience unreliable supply or blackouts in your area? Guidance to provide You need to explain what arrangements are available during power outages and blackouts: - have a backup PSTN phone that is powered by the telephone line - have an alternate phone like a mobile phone handy - if continued operation of your phone is important for your residence or business during a power outage, the use of a backup power supply (known as ‘UPS’ or ‘Uninterruptible Power Supply’) may need to be considered. A UPS operates like a backup battery and typically lasts for one or two hours. |
| Computers and VoIP Page 6, 10 | Questions to ask Do you have a computer at home and are you intending to use it to make VoIP calls or manage your VoIP account? Guidance to provide VoIP phone services can operate independently from a networked computer in your home. Computers can be used to help in the set up process and to access the VoIP service provider’s website. Unlike a VoIP phone service, using a VoIP program on a computer requires the computer to remain on at all times to receive calls using that VoIP service. |

| ## General safety and privacy/security issues |  |
| --- | --- |
| Thunderstorms and acoustic shrieks Page 24 | Guidance to provide You should inform the user of safety issues when using phones, whether the phones are held in the hand or are headsets. One issue is the use of telephones during thunderstorms and another is the possibility of experiencing acoustic shrieks. Resources Further information may be found in the Telstra White Pages or at www.telstra.com.au/thunderstorms/index.htm |
| Security and privacy Page 24 | Background If using a computer in the home or office, normal security and privacy measures should be considered including the use of log-in passwords, virus protection, provision of a firewall and backing up of address books and other sensitive information. As a service provider, you need to consider other privacy/security issues such as VoIP SPAM (also known as SPIT or SPAM over Internet Telephony), e-security and legal and illegal interception. The Telecommunications Act and Telecommunications Interception and Access Act set out the conditions under which legal interception can be conducted by Carriers and carriage service providers for law enforcement and national security agencies. In general terms, unless exempted, all carriers and carriage service providers must provide facilities, which enable them to execute a warrant for interception and to provide special assistance to law enforcement and security agencies. The interception of a communication passing over a telecommunications system is expressly prohibited, except in certain limited circumstances. Resources DCITA: www.dcita.gov.au/communications_for_consumers/security/e-security/internet_security_essentials_for_small_business/internet_security_essentials IIA (Internet Industry Association): http://security.iia.net.au/australian_resources/managing_security/index.html |

# Faults

| ## How to go about solving faults and problems |  |
| --- | --- |
| Defining the fault Page 23 | Questions: Can you describe the fault that you are experiencing? Have you been through the advice in the VoIP Customer Booklet? Guidance to provide You should provide guidance (e.g. a self-help section with on-line diagrams, diagnostic tools) on your website on how to go about determining what the fault is. To provide customer support that reflects best practice, a technical support phone number should be provided in addition to just email support. The fault-finding process described in the remaining part of this section provides a basic guide for users to follow. Background Chopped up or intermittent speech is more likely on lower broadband speeds or lower quality ISP networks. The service or equipment of the person you are calling may be the cause of the poor quality or noise. The person you are calling may be experiencing worse voice quality than you (because of the slower broadband upload speeds). Resources The forums on Whirlpool at http://forums.whirlpool.net.au/ can provide answers on VoIP issues for those with a higher level of technical expertise. |
| Connection Page 23 | If a call does not connect, or suddenly drops out, hang up and call again. If your VoIP service is not working properly: - check that your internet connection is working properly by browsing the internet from your computer. - check that the power is on in each piece of equipment. - check that all the cables are connected properly. - try turning the power off and on again for each piece of equipment (also called ‘rebooting’, just like a computer). - check that your equipment has been setup correctly. |
| Quality Page 23 | If the voice quality is suffering, for example you are getting noise or distortion in your calls (or the person you are speaking with has problems understanding you): - Check for other activity on your computer or home network, for example downloading files from the internet, using file and music sharing programs, sending emails with large attachments. - If there is irritating noise or ‘static’, it may be due to interference from other nearby devices (maybe noticed if you are using a cordless phone). - Check your handset. If possible, plug in a different handset or trying using your handset on another phone line. - If you are using loudspeakers or handsfree, try using a headset or handset. |
| Further problems Page 24 | If the problem still exists, you can: - Check the fault-finding procedures in your manuals. - If you think it is something to do with your VoIP service, seek assistance from your VoIP service provider. - If you think it is something to do with your broadband service, seek assistance from your ISP. |

# Customer service

| ## Customer service |  |
| --- | --- |
| Providing assistance Page 25 | Guidance to provide You need to ensure that your customer knows how to contact your customer service and when it is available. Background Information about your organisation’s customer service offerings, such as call centre operation times and contact details (e.g. free rate, local rate or premium numbers), customer user manuals, call test numbers etc, need to be readily accessible both to staff and customers, via your website and in customer-provided information. Resources Information on you website and on individual product suppliers’ websites. |
| Terms and conditions Page 25 | Guidance to provide You need to ensure that your customer understands the terms and conditions of the VoIP services that you provide. Are the contractual arrangements to use the telephone service fair and easily understood and readily accessible? Background Communications Alliance has developed Codes of Practice for the industry which provide necessary consumer protections. The Consumer Codes, developed in cooperation with industry representatives and consumers, cover the life cycle of the relationship between service providers and their customers. The Code protections start by ensuring that the advertising and promotional material for marketing of telecommunications goods and services are fair and accurate. Those same protections extend to information about prices, terms and conditions supplied before and after a customer signs up for a service. Contracts themselves are also governed by Code rules covering matters such as ensuring that contractual wording is not confusing and does not contain unfair or hidden conditions. Resources Communications Alliance ‘An introduction to Consumer Codes’ www.commsalliance.com.au/about_us/factsheets ACIF C542:2003 Customer Information on Prices, Terms and Conditions and Billing Industry Code and Code Summary sheet ACIF C620:2005 Consumer Contracts Industry Code and Code Summary sheet ACIF C546:2007 Customer Transfer Industry Code and Code Summary sheet Codes can be found at www.commsalliance.com.au/documents/codes Code Summary sheets can be found at www.commsalliance.com.au/about_us/factsheets ACMA Code information www.acma.gov.au/WEB/STANDARD//pc=PC_2524 DCITA policy and legislation information www.dcita.gov.au/communications_and_technology/policy_and_legislation |

# Billing

| ## Service charges |  |
| --- | --- |
| Costs of a telephone service Page 7 | Questions to ask What types of calls do you make today and how much do you spend on calls? Guidance to provide You will need to establish the existing and future service usage patterns of the inquirer to be able to provide the relevant fees. You should list all the costs categories and provide your rates (and savings): - Upfront costs (activation, equipment ) - Monthly fees - Rates for local, long distance, mobile, international calls, on-net calls. - Rates for premium services (if access to these services are offered) - Exit costs - costs for people calling my VoIP service Background The inquirer may be unaware of their spend, especially if they have multiple phone services and bills (local, international, mobile, phone card). Pricing for PSTN and VoIP services can be very different. Provide information in terms that the inquirer would be familiar with (as used in their existing bills). Keep technical terminology to a minimum. Resources VoIP plan comparison at vOip chOice: www.voipchoice.com.au or at Oz Net Phones at www.ozinternetphones.com |
| Impact on broadband bill Page 16 | Questions to ask Do you have broadband already? If so, what type of broadband (ADSL, cable, wireless or satellite) and what plan are you on (monthly data usage allowance)? What is your computer and broadband usage like now? Guidance to provide If they are already using broadband, you should direct them to their broadband plan to make them aware of their ISP’s data download charges and conditions. For example, if the monthly data usage limit is exceeded, either an excess rate applies or the broadband speed is throttled back (or ‘shaped’). Resources The website of the inquirer’s ISP. |

| ## Contracts and billing |  |
| --- | --- |
| Customer obligations Page 11 | Guidance to provide You need to explain the different relationships with the ISP who is providing the broadband service and a VoIP service provider. You need to explain the differences between plans with and without contracts, explain any contractual obligations (legislation and Codes) and about pre-payment and post-payment. You can explain the different billing options and feature that are available: - on-line (via the internet) - real time billing (via the internet) - bill itemising You need to explain if there are any restrictions on the use of the service (e.g. business use versus residential use). You need to explain contractual arrangements such as minimum contract lengths and cancellation fees. |
| Managing telephony spend Page 25 | Background The accuracy and timeliness of bills are governed by Codes, as are credit assessment practices and the processes used for dealing with customers in financial difficulties. The purpose of the Communications Alliance Guide for a Financial Hardship Policy is to assist CSPs in developing internal policies and processes to manage customers who are experiencing difficulty in paying their accounts as a result of financial hardship, in line with Code requirements. Resources ACIF C541:2006 Credit Management Code and Code Summary sheet Communications Alliance Guide for a Financial Hardship Policy at www.commsalliance.com.au/about_us/factsheets |
| Financial difficulty Page 25 | Background The Credit management Code is to assist CSPs in developing internal policies and processes to manage customers who are experiencing difficulty in paying their accounts as a result of financial hardship, in line with Code requirements. Resources ACIF C541:2006 Credit Management Code and Code summary sheet Guide for a Financial Hardship Policy http://www.commsalliance.com.au/about_us/factsheets |

# Consumer protection issues

| ## Consumer protection |  |
| --- | --- |
| Customer Service Guarantee Page 25 | Background The CSG specifies the times in which standard telephone services must be installed and repaired (see link below for more information). How the CSG applies to VoIP services is under consideration by DCITA. Resources ACMA has produced a brochure and FAQ www.acma.gov.au/WEB/STANDARD//pc=PC_2017 |
| Privacy Page 26 | Background The Telecommunications Act 1997 contains a number of provisions dealing with the privacy of personal information held by carriers, carriage service providers and others. Further information regarding privacy and the Telecommunications Act can be found on the Telecommunications page of the Office of the Privacy Commissioner web site. The private sector provisions of the Privacy Act centre around 10 National Privacy Principles (the NPPs) that set out how private sector organisations should collect, use, keep secure and disclose personal information. The principles give individuals a right to know what information an organisation holds about them and the right to correct that information if it is wrong. The Privacy Commissioner has written Guidelines to the National Privacy Principles to assist organisations to meet their obligations in the handling of personal information. A series of information sheets has also been developed and provides more detailed explanations and good practice or compliance tips on various aspects of the National Privacy Principles and the Private Sector provisions. Under the NPP “Openness”, an organisation must set out in a document clearly expressed policies on its management of personal information. The organisation must make the document available to anyone who asks for it. Resources The Office of the Privacy Commissioner www.privacy.gov.au/business/index.html www.privacy.gov.au/act/telecom/index.html ACMA Privacy & your phone service fact sheet http://www.acma.gov.au/WEB/STANDARD//pc=PC_1686 |
| Complaints Page 26 | Guidance to provide You need to ensure that your customer knows what your complaint handling policy is, how to lodge a complaint and the services that the TIO provides. Resources ACIF C547:2004 Complaint Handling Code and Industry brochure |
| Telecommunications Industry Ombudsman (TIO) Page 26 | Background If you provide or resell telephone, mobile or internet services to small business or residential customer, you are required by law (Telecommunications Consumer Protection and Service Standards Act 1999) to be a member of the TIO Scheme. The TIO is a free and independent alternative dispute resolution scheme for small business and residential consumers in Australia with unresolved complaints about their telephone or internet services. It is an office of last resort. Resources The Telecommunications Industry Ombudsman www.tio.com.au |
| Telephone sex services Page 26 | Background If you supply a telephone sex service, you need to understand your obligations under Part 9A of the Telecommunications (Consumer Protection and Services Standards) Act 1999. Resources Restricted access arrangements for telephone sex services www.acma.gov.au/WEB/STANDARD//pc=PC_484 |
| Life threatening or unwelcome calls Page 26 | Background To be better able to assist the community and customers, service providers need to be able to resolve issues of unwelcome calls and to provide assistance in life threatening situations in an efficient and expedient manner. There is a Code which assists Carriers and Carriage Service Providers to define processes that help them work with end users and law enforcement agencies to address life threatening and unwelcome calls. Resources ACIF C525:2006 Handling of Life Threatening and Unwelcome Calls Industry Code |

| General links | Communications Alliance Home: www.commsalliance.com.au VoIP & NGN: www.commsalliance.com.au/Activities/voip ACMA Home: www.acma.gov.au Internet calls: www.acma.gov.au/WEB/STANDARD//pc=PC_100550 VoIP regulation: www.acma.gov.au/WEB/STANDARD//pc=PC_310067 ACCC Home: www.accc.gov.au DCITA Home: www.dcita.gov.au TIO Home: www.tio.com.au Other Voice over IP: en.wikipedia.org/wiki/Voice_over_IP Comparison of VoIP software: en.wikipedia.org/wiki/Comparison_of_VoIP_software |
| --- | --- |
| Communications Alliance Publications | In addition to the Codes and Standards, Communications Alliance publishes the following material, available at http://www.commsalliance.com.au/about_us/factsheets VoIP What you should tell your customers about their Internet Telephone/ VoIP Service Access to emergency services for users of VoIP and Internet Telephony VoIP Security – what you can do about it as a VoIP or Internet Service Provider A basic guide to VoIP technical terms and issues VoIP Customer Booklet Industry Financial Hardship Guide TIO - Your obligations as a Service Provider TIO - Your obligations as a VoIP Service Provider Complaint Handling - Your obligations as a Service Provider Some facts about short messaging to 13/1300 numbers for corporates Consumer An Introduction to Consumer Codes Your Telecommunications Service - As a consumer, what you are entitled to know Some facts about short messaging to 13/1300 numbers for consumers Code Summary Sheets C521:2004 Customer Information On Prices, Terms And Conditions C541:2006 Credit Management C546:2005 Customer Transfer C620:2005 Consumer Contracts C625:2005 and G627:2005 Information Accessibility |

Index

000, 18

106, 13, 18

ABN number, 18

acoustic shrieks, 21

alarm diallers, 7

assurance of service, 17, 18

ATA, 5, 9

bandwidth, 15

billing, 27, 28

blackouts, 21

broadband, 15

broadband plans, 15

business services, 5

call barring, 4

call centre, 5, 25

call costs, 27

call forwarding, 4

call waiting, 4

Calling Number Display, 4, 13

complaints, 29

computers, 19, 21

conference calls, 4

confidentiality, 6

configuration, 19

consumer protection, 29

contracts, 18, 28

cooling-off periods, 18

customer hotline, 19

customer obligations, 28

customer service, 25

Customer Service Guarantee, 29

data usage, 16

delay, 18

dial tones, 5

directory assistance, 14

directory listings, 14

dispute resolution, 30

download speed, 15

drop-outs, 18

echo, 18

emergency calls, 12, 18

EPTPOS, 5

equipment, 5, 19

extension numbers, 5

faults, 23

fax, 4, 7

financial hardship, 28

firewall, 21

free phone numbers, 11

handset, 23

handset quality, 20

home alarm systems, 7

home banking, 7

instant messaging, 4

latency, 15

life threatening calls, 30

line quality, 15

Local Number Portability, 10

local rate numbers, 11

location identification, 6, 13

location independent communications service, 10

locked equipment, 19

manuals, 24

medical monitoring, 8

National Privacy Principles, 29

National Relay Service, 9, 13

networking, 6

noise, 23

nomadicity, 4, 11

Number Plan, 11

numbers, 10

on-net calls, 5, 11

operator-assisted calls, 14

optimisation, 20

passwords, 21

porting, 10, 13, 14

POTS, 4

power outages, 21

Power supplies, 21

premium rate numbers, 11

pre-selection, 12

presence, 4, 6

prioritisation, 20

priority assistance, 8

privacy, 6, 21, 29

provisioning, 4

PSTN, 12, 19, 21

QoS, 16, 18, 20, 23

real-time text, 4

security, 6, 7, 21

Service features, 4

set top boxes, 8

speed test, 16

SPIT, 21

technical support, 23

telephone exchange, 15

telephone line provisioning, 4

telephone outlet, 19

Telephone sex services, 30

teletypewriters, 9

teleworking, 5, 6

terms and conditions, 25

text over IP, 9

textphone, 9

thunderstorms, 21

TIO, 30

trial periods, 18

uninterruptible Power Supply, 21

unlimited plan, 16

unlisted numbers, 14

unwelcome calls, 30

upload speed, 15

video calls, 4

video conferencing, 5

virus protection, 21

voicemail, 4

VoIP phones, 5

Overview of Communications Alliance

Communications Alliance was formed in 2006 from the merger of the Australian Communications Industry Forum (ACIF) and the Service Providers Association (SPAN) to provide a unified voice for the Australian communications industry and to lead it into the next generation of converging networks, technologies and services.

In pursuing its goals, Communications Alliance offers a forum for the industry to make coherent and constructive contributions to policy development and debate.

Communications Alliance seeks to facilitate open, effective and ethical competition between service providers while ensuring efficient, safe operation of networks, the provision of innovative services and the enhancement of consumer outcomes.

It is committed to the achievement of the policy objective of the Telecommunications Act 1997 - the greatest practicable use of industry self-regulation without imposing undue financial and administrative burdens on industry.

Communications Alliance work in VoIP

Communications Alliance is actively involved in assisting the Australian telecommunications industry in the introduction of services such as VoIP. Our Working Committees and Groups are working on many activities including quality of service (QoS) for VoIP and QoS for the Australian IP networks.

The information in this Guide reflects the telecommunications environment and the types of VoIP services being provided at the time of publication. It is not intended to pre-empt any of the outcomes of the work being carried out.

Published by:

COMMUNICATIONS ALLIANCE LTD

Level 9, 32 Walker Street, North Sydney, NSW 2060 Australia

Mailing address: PO Box 444, Milsons Point, NSW 1565

T 61 2 9959 9111	
F 61 2 9954 6136	
TTY 61 2 9923 1911	
E info@commsalliance.com.au 	
www.commsalliance.com.au 	
ABN 56 078 026 507

Copyright 2007

July 2007, 1st edition